# PHẦN 1: CÁC LỆNH HỆ THỐNG

## 1.1. Kết nối SSH vào BeagleBone Black

# Ket noi SSH qua USB (ip mac dinh)

ssh debian@192.168.7.2

# Mat khau mac dinh

temppwd

## 1.2. Copy file từ Ubuntu lên BBB

# Copy toan bo thu muc project

scp -r /home/manhht10ht/do_an_nhung/ debian@192.168.7.2:/home/debian/

# Copy file don le

scp project_v4.c debian@192.168.7.2:/home/debian/do_an_nhung/

# Copy file tu BBB ve Ubuntu

scp debian@192.168.7.2:/home/debian/do_an_nhung/project_v4 ./

## 1.3. Chia sẻ internet từ Ubuntu sang BBB qua USB

# Tren Ubuntu - bat ip forward va NAT

sudo sysctl net.ipv4.ip_forward=1

sudo iptables -t nat -A POSTROUTING -o enp0s3 -j MASQUERADE

# Tren BBB - them default route va DNS

sudo ip route add default via 192.168.7.1

echo 'nameserver 8.8.8.8' | sudo tee /etc/resolv.conf

# Kiem tra ket noi

ping 8.8.8.8 -c 3

## 1.4. Cài đặt các gói cần thiết trên BBB

# Cap nhat danh sach goi

sudo apt update

# Cai kernel headers de build driver

sudo apt install linux-headers-$(uname -r) build-essential -y

# Cai cac cong cu khac

sudo apt install gdbserver i2c-tools curl -y

## 1.5. Tắt BBB an toàn

sudo shutdown -h now

# PHẦN 2: BUILD VÀ NẠP KERNEL DRIVER

## 2.1. Build tất cả driver

cd /home/debian/do_an_nhung/

make clean

make

## 2.2. Kiểm tra thiết bị I2C

# Quet I2C bus 1

sudo i2cdetect -y -r 1

# Quet I2C bus 2 (bus chinh dung trong du an)

sudo i2cdetect -y -r 2

# Ket qua mong doi:

# 0x27 = LCD I2C

# 0x48 = ADS1115 (MQ2)

## 2.3. Nạp driver theo thứ tự

# 1. Nap DHT11 driver (GPIO)

sudo insmod dht11_driver.ko

sudo chmod 666 /dev/dht11

# 2. Nap Buzzer driver (GPIO)

sudo insmod buzzer_driver.ko

MAJOR=$(grep ' buzzer' /proc/devices | awk '{print $1}')

sudo mknod /dev/buzzer c $MAJOR 0

sudo chmod 666 /dev/buzzer

echo 1 | sudo tee /dev/buzzer  # Tat coi mac dinh

# 3. Nap ADS1115 driver (I2C - MQ2)

echo '2-0048' | sudo tee /sys/bus/i2c/drivers/ads1015/unbind 2>/dev/null

echo 'ads1115 0x48' | sudo tee /sys/bus/i2c/devices/i2c-2/new_device

sudo insmod ads1115_driver.ko

MAJOR=$(grep 'mq2' /proc/devices | awk '{print $1}')

sudo mknod /dev/mq2 c $MAJOR 0

sudo chmod 666 /dev/mq2

# 4. Nap LCD driver (I2C)

echo 'i2c_lcd 0x27' | sudo tee /sys/bus/i2c/devices/i2c-2/new_device

sudo insmod lcd_driver.ko

MAJOR=$(grep ' lcd' /proc/devices | awk '{print $1}')

sudo mknod /dev/lcd c $MAJOR 0

sudo chmod 666 /dev/lcd

## 2.4. Kiểm tra driver sau khi nạp

# Xem log kernel

dmesg | tail -20

# Xem danh sach device node

ls -la /dev/dht11 /dev/buzzer /dev/mq2 /dev/lcd

# Xem danh sach tat ca character device

cat /proc/devices

## 2.5. Gỡ driver

sudo rmmod lcd_driver

sudo rmmod ads1115_driver

sudo rmmod dht11_driver

sudo rmmod buzzer_driver

## 2.6. Test từng cảm biến

# Test DHT11 - doc nhiet do va do am

./app

# Test Buzzer

echo 0 > /dev/buzzer   # Bat coi (Active LOW)

echo 1 > /dev/buzzer   # Tat coi

# Test MQ2 - doc gia tri ADC

cat /dev/mq2

# Test LCD - hien thi chu

echo 'Hello BBB!' > /dev/lcd

echo -e 'Dong 1\nDong 2' > /dev/lcd

# PHẦN 3: QUẢN LÝ SYSTEMD SERVICE

## 3.1. Tạo file service

# Tao file service

sudo nano /etc/systemd/system/pccc.service

## 3.2. Nội dung file pccc.service

[Unit]

Description=He thong canh bao PCCC

After=network.target

[Service]

Type=simple

User=root

Environment=HOME=/root

WorkingDirectory=/home/debian/do_an_nhung

ExecStart=/home/debian/do_an_nhung/start.sh

Restart=on-failure

RestartSec=5

[Install]

WantedBy=multi-user.target

## 3.3. Các lệnh quản lý service

# Kich hoat tu dong khi boot

sudo systemctl enable pccc.service

# Tai lai cau hinh sau khi sua file service

sudo systemctl daemon-reload

# Khoi dong service

sudo systemctl start pccc.service

# Dung service

sudo systemctl stop pccc.service

# Khoi dong lai service

sudo systemctl restart pccc.service

# Kiem tra trang thai

sudo systemctl status pccc.service

# Xem log real-time

sudo journalctl -u pccc.service -f

# Xem log gan day

sudo journalctl -u pccc.service -n 50

# Test tu phuc hoi - tim PID va kill

sudo systemctl status pccc.service | grep 'Main PID'

sudo kill -9 <PID>

# Sau 5 giay service tu dong restart

# PHẦN 4: GDB REMOTE DEBUG

## 4.1. Chuẩn bị

# Tren BBB - cai gdbserver

sudo apt install gdbserver -y

# Tren Ubuntu - cai gdb-multiarch

sudo apt install gdb-multiarch -y

## 4.2. Compile với debug symbols

# Compile voi flag -g de co debug symbols

gcc -g -o project_v4 project_v4.c -lpthread

## 4.3. Khởi động GDB Server trên BBB

# Chay chuong trinh qua gdbserver, lang nghe port 2345

gdbserver :2345 ./project_v4

# Ket qua mong doi:

# Process ./project_v4 created; pid = XXXX

# Listening on port 2345

## 4.4. Kết nối debug từ Ubuntu

# Mo gdb tren Ubuntu voi file binary

gdb-multiarch ./project_v4

# Ket noi vao BBB

(gdb) target remote 192.168.7.2:2345

# Dat breakpoint tai ham thread_print

(gdb) break thread_print

# Chay chuong trinh den breakpoint

(gdb) continue

# Xem gia tri bien khi dung tai breakpoint

(gdb) print temp

(gdb) print humi

(gdb) print gas

# Tiep tuc chay

(gdb) continue

# Thoat gdb

(gdb) quit

## 4.5. Các lệnh GDB thường dùng

# Dat breakpoint theo dong code

(gdb) break project_v4.c:89

# Xem danh sach tat ca thread

(gdb) info threads

# Chuyen sang thread khac

(gdb) thread 2

# Chay tung dong

(gdb) next

(gdb) step

# Xem stack trace

(gdb) backtrace

# Xoa breakpoint

(gdb) delete 1

# PHẦN 5: BLYNK IOT

## 5.1. Gửi dữ liệu lên Blynk (test thủ công)

# Cap nhat gia tri don le

curl 'https://blynk.cloud/external/api/update?token=<TOKEN>&V0=30'

# Cap nhat nhieu gia tri cung luc (batch update)

curl 'https://blynk.cloud/external/api/batch/update?token=<TOKEN>&V0=30&V1=45&V2=1000'

## 5.2. Gửi event cảnh báo

# Gui event fire_alert khi phat hien chay

curl 'https://blynk.cloud/external/api/logEvent?token=<TOKEN>&code=fire_alert'

## 5.3. Thông tin Blynk của project

# BLYNK_TEMPLATE_ID: TMPL6ubsTur6C

# BLYNK_TEMPLATE_NAME: PCCC

# BLYNK_AUTH_TOKEN: 2tNHijXeZqOhXD7LAiUM2DXVJPNUG18Y

# V0 = Temperature (nhiet do)

# V1 = Humidity (do am)

# V2 = Gas (gia tri ADC MQ2)

# PHẦN 6: TOÀN BỘ SOURCE CODE

## 6.1. Makefile

obj-m += dht11_driver.o buzzer_driver.o ads1115_driver.o lcd_driver.o

all:

make -C /lib/modules/$(shell uname -r)/build M=$(PWD) modules

clean:

make -C /lib/modules/$(shell uname -r)/build M=$(PWD) clean

## 6.2. dht11_driver.c

#include <linux/module.h>

#include <linux/init.h>

#include <linux/gpio.h>

#include <linux/gpio/consumer.h>

#include <linux/delay.h>

#include <linux/irqflags.h>

#include <linux/fs.h>

#include <linux/uaccess.h>

#include <linux/device.h>

#define DHT11_PIN 45

#define DEVICE_NAME "dht11"

static int major_number;

static struct class* dht11Class = NULL;

static struct device* dht11Device = NULL;

struct gpio_desc *dht11_desc;

static uint8_t dht11_data[5] = {0};

static int read_dht11_raw_data(void) {

uint8_t i, j;

int counter = 0;

unsigned long flags;

for(i=0; i<5; i++) dht11_data[i] = 0;

gpiod_direction_output(dht11_desc, 0);

mdelay(18);

gpiod_set_value(dht11_desc, 1);

udelay(30);

gpiod_direction_input(dht11_desc);

local_irq_save(flags);

counter = 0;

while (gpiod_get_value(dht11_desc)==1){counter++;udelay(1);if(counter>100)break;}

counter = 0;

while (gpiod_get_value(dht11_desc)==0){counter++;udelay(1);if(counter>100)break;}

counter = 0;

while (gpiod_get_value(dht11_desc)==1){counter++;udelay(1);if(counter>100)break;}

for (i = 0; i < 5; i++) {

for (j = 0; j < 8; j++) {

counter = 0;

while(gpiod_get_value(dht11_desc)==0){counter++;udelay(1);if(counter>100)break;}

counter = 0;

while(gpiod_get_value(dht11_desc)==1){counter++;udelay(1);if(counter>100)break;}

dht11_data[i] <<= 1;

if (counter > 40) dht11_data[i] |= 1;

}

}

local_irq_restore(flags);

if((uint8_t)(dht11_data[0]+dht11_data[1]+dht11_data[2]+dht11_data[3])==dht11_data[4])

return 0;

return -1;

}

static ssize_t dev_read(struct file *filep, char *buffer, size_t len, loff_t *offset) {

int ret, i;

for(i = 0; i < 10; i++) {

if (read_dht11_raw_data() == 0) {

if(dht11_data[0] > 0 && dht11_data[2] > 0) {

ret = copy_to_user(buffer, dht11_data, 5);

return (ret == 0) ? 5 : -EFAULT;

}

}

mdelay(100);

}

return -EAGAIN;

}

static struct file_operations fops = { .read = dev_read };

static int __init dht11_init(void) {

if (gpio_request(DHT11_PIN, "DHT11_DATA") < 0) return -EBUSY;

dht11_desc = gpio_to_desc(DHT11_PIN);

gpiod_direction_input(dht11_desc);

major_number = register_chrdev(0, DEVICE_NAME, &fops);

dht11Class = class_create(THIS_MODULE, "dht11_class");

dht11Device = device_create(dht11Class, NULL, MKDEV(major_number,0), NULL, DEVICE_NAME);

printk(KERN_INFO "DHT11 Driver Loaded! Device: /dev/%s\n", DEVICE_NAME);

return 0;

}

static void __exit dht11_exit(void) {

device_destroy(dht11Class, MKDEV(major_number, 0));

class_destroy(dht11Class);

unregister_chrdev(major_number, DEVICE_NAME);

gpio_free(DHT11_PIN);

printk(KERN_INFO "DHT11 Driver Removed!\n");

}

module_init(dht11_init);

module_exit(dht11_exit);

MODULE_LICENSE("GPL");

## 6.3. buzzer_driver.c

#include <linux/module.h>

#include <linux/init.h>

#include <linux/gpio.h>

#include <linux/fs.h>

#include <linux/uaccess.h>

#include <linux/device.h>

#define GPIO_BUZZER 23

#define DEVICE_NAME "buzzer"

static int major;

static struct class *buzzer_class;

static struct device *buzzer_device;

static ssize_t dev_write(struct file *file, const char __user *buf,

size_t len, loff_t *off) {

char kbuf[2] = {0};

if (copy_from_user(kbuf, buf, 1)) return -EFAULT;

if (kbuf[0] == '0') {

gpio_set_value(GPIO_BUZZER, 0);

printk(KERN_INFO "Buzzer Driver: COI DANG KEU\n");

} else {

gpio_set_value(GPIO_BUZZER, 1);

printk(KERN_INFO "Buzzer Driver: COI DA TAT\n");

}

return len;

}

static struct file_operations fops = { .write = dev_write };

static int __init buzzer_init(void) {

if (gpio_request(GPIO_BUZZER, "BUZZER_GPIO")) return -EBUSY;

gpio_direction_output(GPIO_BUZZER, 1);

major = register_chrdev(0, DEVICE_NAME, &fops);

buzzer_class = class_create(THIS_MODULE, "buzzer_class");

buzzer_device = device_create(buzzer_class, NULL, MKDEV(major,0), NULL, DEVICE_NAME);

printk(KERN_INFO "Buzzer Driver: Nap thanh cong. Major = %d\n", major);

return 0;

}

static void __exit buzzer_exit(void) {

gpio_set_value(GPIO_BUZZER, 1);

gpio_free(GPIO_BUZZER);

device_destroy(buzzer_class, MKDEV(major, 0));

class_destroy(buzzer_class);

unregister_chrdev(major, DEVICE_NAME);

}

module_init(buzzer_init);

module_exit(buzzer_exit);

MODULE_LICENSE("GPL");

## 6.4. ads1115_driver.c

#include <linux/module.h>

#include <linux/i2c.h>

#include <linux/fs.h>

#include <linux/uaccess.h>

#include <linux/delay.h>

static struct i2c_client *ads_client;

static int major;

static ssize_t dev_read(struct file *filep, char *buffer, size_t len, loff_t *offset) {

uint8_t config[3] = {0x01, 0xC3, 0x83};

uint8_t reg = 0x00;

uint8_t data[2];

char msg[16];

i2c_master_send(ads_client, config, 3);

mdelay(10);

i2c_master_send(ads_client, &reg, 1);

i2c_master_recv(ads_client, data, 2);

int16_t value = (data[0] << 8) | data[1];

int n = sprintf(msg, "%d\n", value);

return copy_to_user(buffer, msg, n) ? -EFAULT : n;

}

static struct file_operations fops = { .read = dev_read };

static int ads_probe(struct i2c_client *client, const struct i2c_device_id *id) {

ads_client = client;

major = register_chrdev(0, "mq2", &fops);

printk(KERN_INFO "MQ2 Driver: Da nhan ADS1115 tai 0x%x. Major: %d\n",

client->addr, major);

return 0;

}

static int ads_remove(struct i2c_client *client) {

unregister_chrdev(major, "mq2");

return 0;

}

static const struct i2c_device_id ads_ids[] = { {"ads1115", 0}, {} };

static struct i2c_driver ads_driver = {

.driver = { .name = "ads1115" },

.probe = ads_probe,

.remove = ads_remove,

.id_table = ads_ids

};

module_i2c_driver(ads_driver);

MODULE_LICENSE("GPL");

## 6.5. lcd_driver.c

#include <linux/module.h>

#include <linux/i2c.h>

#include <linux/fs.h>

#include <linux/uaccess.h>

#include <linux/delay.h>

#define DEVICE_NAME "lcd"

static int major;

static struct i2c_client *lcd_client;

void lcd_send_4bit(u8 val) {

u8 buf;

buf = val | 0x0C;

i2c_master_send(lcd_client, &buf, 1);

usleep_range(1000, 2000);

buf = val | 0x08;

i2c_master_send(lcd_client, &buf, 1);

usleep_range(1000, 2000);

}

void lcd_send_byte(u8 val, u8 mode) {

lcd_send_4bit((val & 0xf0) | mode);

lcd_send_4bit(((val << 4) & 0xf0) | mode);

}

static ssize_t dev_write(struct file *file, const char __user *buf,

size_t len, loff_t *off) {

char kbuf[32];

int i, n = (len > 32) ? 32 : len;

if (copy_from_user(kbuf, buf, n)) return -EFAULT;

lcd_send_byte(0x01, 0x00); msleep(5);

for (i = 0; i < n; i++) {

if (kbuf[i] == '\n') { lcd_send_byte(0xC0, 0x00); msleep(2); continue; }

lcd_send_byte(kbuf[i], 0x01);

usleep_range(500, 1000);

}

return len;

static struct file_perations fops = { .write = dev_write };

static int lcd_probe(struct i2c_client *client, const struct i2c_device_id *id) {

lcd_client = client;

major = register_chrdev(0, DEVICE_NAME, &fops);

msleep(50);

lcd_send_4bit(0x30); msleep(5);

lcd_send_4bit(0x30); msleep(2);

lcd_send_4bit(0x30); msleep(2);

lcd_send_4bit(0x20); msleep(2);

lcd_send_byte(0x28, 0x00);

lcd_send_byte(0x0C, 0x00);

lcd_send_byte(0x06, 0x00);

lcd_send_byte(0x01, 0x00); msleep(5);

printk(KERN_INFO "LCD Driver: Probe thanh cong!\n");

return 0;

}

static int lcd_remove(struct i2c_client *client) {

unregister_chrdev(major, DEVICE_NAME);

return 0;

}

static const struct i2c_device_id lcd_ids[] = { {"i2c_lcd", 0}, {} };

static struct i2c_driver lcd_driver = {

.driver = { .name = "i2c_lcd" },

.probe = lcd_probe,

.remove = lcd_remove,

.id_table = lcd_ids

};

module_i2c_driver(lcd_driver);

MODULE_LICENSE("GPL");

## 6.6. project_v4.c - Ứng dụng chính đa luồng + Blynk

#include <stdio.h>

#include <stdlib.h>

#include <fcntl.h>

#include <unistd.h>

#include <string.h>

#include <pthread.h>

#define BLYNK_TOKEN "2tNHijXeZqOhXD7LAiUM2DXVJPNUG18Y"

int temp = 0, humi = 0, gas = 0;

pthread_mutex_t mutex = PTHREAD_MUTEX_INITIALIZER;

void *thread_dht11(void *arg) {

while(1) {

int fd = open("/dev/dht11", O_RDONLY);

if (fd >= 0) {

unsigned char data[5] = {0};

if (read(fd, data, 5) == 5) {

if (data[0] > 0 && data[2] > 0 && data[0] <= 100 && data[2] <= 50) {

pthread_mutex_lock(&mutex);

humi = data[0]; temp = data[2];

pthread_mutex_unlock(&mutex);

}

}

close(fd);

}

sleep(2);

}

return NULL;

}

void *thread_mq2(void *arg) {

char buf[16];

while(1) {

int fd = open("/dev/mq2", O_RDONLY);

if (fd >= 0) {

memset(buf, 0, 16);

if (read(fd, buf, 15) > 0) {

pthread_mutex_lock(&mutex);

gas = atoi(buf);

pthread_mutex_unlock(&mutex);

}

close(fd);

}

sleep(1);

}

return NULL;

}

void *thread_lcd(void *arg) {

while(1) {

int fd = open("/dev/lcd", O_WRONLY);

if (fd >= 0) {

char msg[32];

pthread_mutex_lock(&mutex);

snprintf(msg, sizeof(msg), "T:%dC H:%d%%\nGas:%d", temp, humi, gas);

pthread_mutex_unlock(&mutex);

write(fd, msg, strlen(msg));

close(fd);

}

sleep(1);

}

return NULL;

}

void *thread_buzzer(void *arg) {

while(1) {

int fd = open("/dev/buzzer", O_WRONLY);

if (fd >= 0) {

pthread_mutex_lock(&mutex);

int g = gas;

pthread_mutex_unlock(&mutex);

if (g > 4000) write(fd, "0", 1);

else write(fd, "1", 1);

close(fd);

}

sleep(1);

}

return NULL;

}

void *thread_print(void *arg) {

while(1) {

pthread_mutex_lock(&mutex);

printf("Running: T:%d H:%d Gas:%d\n", temp, humi, gas);

pthread_mutex_unlock(&mutex);

sleep(2);

}

return NULL;

}

void *thread_blynk(void *arg) {

char cmd[512];

while(1) {

pthread_mutex_lock(&mutex);

int t = temp, h = humi, g = gas;

pthread_mutex_unlock(&mutex);

snprintf(cmd, sizeof(cmd),

"curl -s \"https://blynk.cloud/external/api/batch/update"

"?token=" BLYNK_TOKEN

"&V0=%d&V1=%d&V2=%d\"", t, h, g);

system(cmd);

printf("Sent to Blynk: T:%d H:%d Gas:%d\n", t, h, g);

if (g > 4000) {

snprintf(cmd, sizeof(cmd),

"curl -s \"https://blynk.cloud/external/api/logEvent"

"?token=" BLYNK_TOKEN "&code=fire_alert\"");

system(cmd);

printf("FIRE ALERT sent to Blynk!\n");

}

sleep(1);

}

return NULL;

}

int main() {

pthread_t t1, t2, t3, t4, t5, t6;

pthread_create(&t1, NULL, thread_dht11, NULL);

pthread_create(&t2, NULL, thread_mq2, NULL);

pthread_create(&t3, NULL, thread_lcd, NULL);

pthread_create(&t4, NULL, thread_buzzer, NULL);

pthread_create(&t5, NULL, thread_print, NULL);

pthread_create(&t6, NULL, thread_blynk, NULL);

pthread_join(t1, NULL); pthread_join(t2, NULL);

pthread_join(t3, NULL); pthread_join(t4, NULL);

pthread_join(t5, NULL); pthread_join(t6, NULL);

return 0;

}

## 6.7. start.sh - Script khởi động

#!/bin/bash

cd /home/debian/do_an_nhung/

echo '=== Nap DHT11 driver ==='

sudo insmod dht11_driver.ko 2>/dev/null

sudo chmod 666 /dev/dht11

echo '=== Nap Buzzer driver ==='

sudo insmod buzzer_driver.ko 2>/dev/null

MAJOR=$(grep ' buzzer' /proc/devices | awk '{print $1}')

[ ! -e /dev/buzzer ] && sudo mknod /dev/buzzer c $MAJOR 0

sudo chmod 666 /dev/buzzer

echo 1 | sudo tee /dev/buzzer > /dev/null

echo '=== Nap ADS1115 driver ==='

echo '2-0048' | sudo tee /sys/bus/i2c/drivers/ads1015/unbind 2>/dev/null

echo 'ads1115 0x48' | sudo tee /sys/bus/i2c/devices/i2c-2/new_device 2>/dev/null

sudo rmmod ads1115_driver 2>/dev/null

sudo insmod ads1115_driver.ko 2>/dev/null

sleep 1

MAJOR=$(grep 'mq2' /proc/devices | awk '{print $1}')

[ ! -e /dev/mq2 ] && sudo mknod /dev/mq2 c $MAJOR 0

sudo chmod 666 /dev/mq2

echo '=== Reset LCD ==='

echo '0x27' | sudo tee /sys/bus/i2c/devices/i2c-2/delete_device 2>/dev/null

sleep 1

echo '=== Nap LCD driver ==='

echo 'i2c_lcd 0x27' | sudo tee /sys/bus/i2c/devices/i2c-2/new_device 2>/dev/null

sudo rmmod lcd_driver 2>/dev/null

sudo insmod lcd_driver.ko 2>/dev/null

sleep 1

MAJOR=$(grep ' lcd' /proc/devices | awk '{print $1}')

[ ! -e /dev/lcd ] && sudo mknod /dev/lcd c $MAJOR 0

sudo chmod 666 /dev/lcd

echo '=== Chay chuong trinh chinh ==='

./project_v4

## 6.8. Lệnh compile ứng dụng

# Compile thong thuong

gcc -o project_v4 project_v4.c -lpthread

# Compile voi debug symbols

gcc -g -o project_v4 project_v4.c -lpthread